Chamber-Aware Surrogate Modeling and Data Assimilation for Semiconductor Plasma Process Design

Author Name1 and Co-author Name1

1 Tokyo Electron Ltd., Minato-ku, Tokyo 107-6325, Japan

Email: corresponding.author@example.com

Plasma processes remain central to semiconductor manufacturing, where nanometer-scale control is increasingly required for advanced three-dimensional device architectures [1]. In plasma equipment development, the challenge extends beyond sustaining stable discharges to identifying chamber geometries and process windows that reproducibly realize target plasma states. Plasma simulation is therefore an indispensable tool for reactor design and process development. However, chamber-scale simulations require careful selection of physical approximations, reaction mechanisms, and collision cross-section data, while their strong nonlinearity makes parametric studies computationally expensive [2].

Recent advances in data-driven plasma science have demonstrated the potential of physics-aware machine learning to accelerate plasma analysis while preserving physical interpretability [3–5]. In parallel, Tokyo Electron has been advancing process informatics and digital twin initiatives, including virtual metrology and machine-learning-assisted process development [6,7].

In this work, we present a chamber-aware surrogate modeling framework for plasma simulations in semiconductor processing reactors. The proposed framework extracts compact features from simulation fields while retaining chamber-specific boundary conditions and hardware-dependent physical structure. It predicts spatial distributions of key plasma quantities, including electron density, ion density, electron temperature, and plasma potential, from process and design variables such as gas composition, pressure, RF power, bias conditions, and chamber configuration.

We further discuss two major applications of the trained surrogate model. First, it enables high-throughput exploration of design and operating conditions that would be impractical with the original plasma solver alone. Second, when combined with sparse observations, it provides a practical framework for data assimilation and inverse design, in which target plasma indicators are specified and the corresponding process conditions or design parameters are estimated under equipment constraints.

Thus, the surrogate model is valuable not merely as a computational substitute for simulation, but as an interface layer linking physics-based modeling, limited measurements, and manufacturing decision-making. The overall aim is to shorten development lead time while improving the reproducibility and transferability of optimized plasma conditions to real semiconductor manufacturing tools.References

References

[1] G. S. Oehrlein et al., "Future of plasma etching for microelectronics," J. Vac. Sci. Technol. B 42, 041501 (2024)

[2] E. Carbone, W. Graef, G. J. M. Hagelaar et al., "Data Needs for Modeling Low-Temperature Non-Equilibrium Plasmas," Atoms 9, 16 (2021)

[3] R. Anirudh et al., "2022 Review of Data-Driven Plasma Science," IEEE Trans. Plasma Sci. 51, 1750-1838 (2023)

[4] J. Trieschmann, L. Vialetto and T. Gergs, "Machine learning for advancing low-temperature plasma modeling and simulation," J. Micro/Nanopattern. Mater. Metrol. 22, 041504 (2023)

[5] L. Zhong, B. Wu and Y. Wang, "Low-temperature plasma simulation based on physics-informed neural networks," Phys. Fluids 34, 087116 (2022)

[6] T. Tsutsui and T. Matsuzawa, "Virtual Metrology Model Robustness Against Chamber Condition Variation Using Deep Learning," IEEE Trans. Semicond. Manuf. 32, 428-433 (2019)

[7] T. Moriya, "Utilization of machine learning in the semiconductor manufacturing process," Oyo Buturi 90, 290-293 (2021)